Certificate of Analysis

31.01.2025

PineChemical Group Oy

Helsinki, Finland

SULPHATE TURPENTINE (PTK GRADE)

Specification: Technical conditions 2416-006-18854671-2015

Date of Production: 30.01.2025

Container no.: SEGU8096918

Net weight: 21 200 kgs

Contract no.: Additional Agreement №2 to the Supply Agreement №15/01/2025

| Properties | Test results |
| --- | --- |
| Appearance | Transparent volatile liquid with no residue and water |
| Density at 20°C, g/cm3, in a region of | 0.858 |
| Index of refraction | 1.469 |
| ### Temperature limits of distillation at the pressure of 10/325 Pa (760mm of mercury column) - initial boiling point, °C, min - distillate volume fraction up to 170°C, %, min | ### 155 95 |
| ### Mass fraction of sulfur, % | 0.011 |
| Mass fraction of the residue from evaporation, %, no more than | 0.04 |
| Acid number, mg KOH /g | 0.04 |

Conclusion of quality control department: It is hereby certified that the quality of Sulphate Turpentine (PTK grade) mentioned in this certificate is in conformity with the terms and conditions of the contract. In all applications of Sulphate Turpentine (PTK grade) it is the sole responsibility of the buyer to respect and comply with any valid intellectual property rights of third parties

PineChemical Group OY representative					                    Natalia Sokolova